SPI Slave with Single Port RAM
Project Documentation Report

Prepared by: Abdalrahman Hamdy Ashry

Course: Digital Electronics / RTL Design

| Member Name | Certificate Name | ID / Role |
| --- | --- | --- |
| [Insert Name] | [Insert Certificate Name] | [Insert ID] |
| [Insert Name] | [Insert Certificate Name] | [Insert ID] |
| [Insert Name] | [Insert Certificate Name] | [Insert ID] |

## 1.RTL

A. Single-Port Synchronous RAM

// Single-Port Synchronous RAM

module ram #(

parameter MEM_DEPTH = 256,

parameter ADDR_SIZE = 8

)(

input  wire                 clk,

input  wire                 rst_n,

input  wire [ADDR_SIZE+1:0] din,

input  wire                 rx_valid,

output reg  [ADDR_SIZE-1:0] dout,

output reg                  tx_valid

);

// State encoding for the SPI commands

localparam WRITE_ADDR = 2'b00 ;

localparam WRITE_DATA = 2'b01 ;

localparam READ_ADDR  = 2'b10 ;

localparam READ_DATA  = 2'b11 ;

// RAM memory array

reg [ADDR_SIZE-1:0] RAM [MEM_DEPTH-1:0];

// Address registers for read and write operations

reg [ADDR_SIZE-1:0] rx_addr;

reg [ADDR_SIZE-1:0] wr_addr;

// Sequential logic for handling read and write operations

always @(posedge clk) begin

if (!rst_n) begin                       // reset condition

dout <= {ADDR_SIZE{1'b0}};

tx_valid <= 1'b0;

wr_addr <= {ADDR_SIZE{1'b0}};

rx_addr <= {ADDR_SIZE{1'b0}};

end

else begin

// default value

tx_valid <= 1'b0;

if (rx_valid) begin

case (din[ADDR_SIZE+1:ADDR_SIZE])

WRITE_ADDR : wr_addr <= din[ADDR_SIZE-1:0];         // store the write address

WRITE_DATA : RAM[wr_addr] <= din[ADDR_SIZE-1:0];    // write data to RAM at the stored address

READ_ADDR  : rx_addr <= din[ADDR_SIZE-1:0];         // store the read address

READ_DATA  : begin                                  // read data from RAM at the stored address

dout <= RAM [rx_addr];

tx_valid <= 1'b1;

end

default : ;

endcase

end

end

end

endmodule

B.  SPI Slave FSM Interface

// SPI Slave FSM Interface

module spi_slave #(

parameter ADDR_SIZE = 8

)

(

input  wire       clk,

input  wire       rst_n,

input  wire       SS_n,

input  wire       MOSI,

input  wire [ADDR_SIZE-1:0] tx_data,

input  wire       tx_valid,

output reg  [ADDR_SIZE+1:0] rx_data,

output reg        rx_valid,

output reg        MISO

);

// 1.FSM state encoding

// local parameters for the SPI states

// Sequential logic for the SPI Slave FSM

localparam IDLE     = 3'b000;

localparam CHK_CMD  = 3'b001;

localparam WRITE    = 3'b010;

localparam READ_ADD = 3'b011;

localparam READ_DATA= 3'b100;

// current and next state registers

(* fsm_encoding = "one_hot" *) reg [2:0] curr_state , next_state;

reg [3:0]           bit_counter;

reg [ADDR_SIZE+1:0] rx_shift_reg;

// 10 bit shift register for transmitting data

reg [ADDR_SIZE-1:0] tx_shift_reg;

// Flag to track read address step

reg                 rd_addr_received;

// 1. counter & shift in Logic (MOSI -> rx_data) & shift out Logic (RAM -> MISO)

// (Synchronous Reset)

always @(posedge clk ) begin

if (!rst_n) begin

rx_data         <= {(ADDR_SIZE+2){1'b0}};

rx_shift_reg    <= {(ADDR_SIZE+2){1'b0}};

tx_shift_reg    <= {(ADDR_SIZE){1'b0}};

MISO            <= 1'b0;

bit_counter     <= 4'd0;

rd_addr_received <= 1'b0;

end

else begin

if (SS_n) begin

bit_counter <= 4'd0;

MISO        <= 1'b0;

end

else begin

rx_shift_reg <= {rx_shift_reg[ADDR_SIZE:0],MOSI};

//shift in  MOSI data

if (bit_counter == 4'd9) begin

rx_data     <= {rx_shift_reg[ADDR_SIZE:0],MOSI};

bit_counter <= 4'd0;

if (rx_shift_reg[ADDR_SIZE] == 1'b1 && rx_shift_reg[ADDR_SIZE-1] == 1'b0)

rd_addr_received <= 1'b1;

else if (rx_shift_reg[ADDR_SIZE] == 1'b1 && rx_shift_reg[ADDR_SIZE-1] == 1'b1)

rd_addr_received <= 1'b0;

end

else begin

bit_counter <= bit_counter + 1'b1;

end

// shift out

if (curr_state == READ_DATA) begin

if (tx_valid) begin

MISO         <= tx_data[ADDR_SIZE-1];

tx_shift_reg <= {tx_data[ADDR_SIZE-2:0], 1'b0};

end

else begin

MISO         <= tx_shift_reg[ADDR_SIZE-1];

tx_shift_reg <= {tx_shift_reg[ADDR_SIZE-2:0], 1'b0};

end

end

else begin

MISO <= 1'b0;

end

end

end

end

// 2. STATE MEM

always @(posedge clk ) begin

if (!rst_n) begin

curr_state <= IDLE;

end

else begin

curr_state <= next_state;

end

end

// 3. Next state logic

always @(*) begin

case (curr_state)

IDLE: begin

if (!SS_n)                      next_state = CHK_CMD;

else                            next_state = IDLE;

end

CHK_CMD: begin

if (SS_n)

next_state = IDLE;

else if (bit_counter == 4'd9) begin

// MOSI is sampled on posedge, checking incoming bit:

if (rx_shift_reg[ADDR_SIZE] == 1'b0)

next_state = WRITE;

else begin

if (rx_shift_reg[ADDR_SIZE-1] == 1'b0)

next_state = READ_ADD;

else

next_state = READ_DATA;

end

end

else

next_state = CHK_CMD;

end

WRITE:begin

if(SS_n)                            next_state = IDLE;

else                                next_state = WRITE;

end

READ_ADD:begin

if(SS_n)                            next_state = IDLE;

else                                next_state = READ_ADD;

end

READ_DATA:begin

if(SS_n )                           next_state = IDLE;

else                                next_state = READ_DATA;

end

default:                                next_state = IDLE;

endcase

end

// 4. output logic

always @(posedge clk) begin

if (!rst_n) rx_valid <= 1'b0;

else begin

if (!SS_n && curr_state == CHK_CMD && bit_counter == 4'd9)

rx_valid <= 1'b1;

else                                  rx_valid <= 1'b0;

end

end

endmodule

C. Top Level Wrapper connecting SPI Slave and RAM

// Top Level Wrapper connecting SPI Slave and RAM

module spi_wrapper (

input  wire clk,

input  wire rst_n,

input  wire SS_n,

input  wire MOSI,

output wire MISO

);

wire [9:0] rx_data_wire;

wire       rx_valid_wire;

wire [7:0] tx_data_wire;

wire       tx_valid_wire;

// SPI Slave Instance

spi_slave #(.ADDR_SIZE(8))

u_spi_slave (

.clk(clk),

.rst_n(rst_n),

.SS_n(SS_n),

.MOSI(MOSI),

.rx_data(rx_data_wire),

.rx_valid(rx_valid_wire),

.tx_data(tx_data_wire),

.tx_valid(tx_valid_wire),

.MISO(MISO)

);

// Single Port RAM Instance

ram #(.MEM_DEPTH(256),

.ADDR_SIZE(8))

u_ram (

.clk(clk),

.rst_n(rst_n),

.din(rx_data_wire),

.rx_valid(rx_valid_wire),

.dout(tx_data_wire),

.tx_valid(tx_valid_wire)

);

endmodule

Run.do

# --- 1. Create and map work library ---

vlib work

vmap work work

# --- 2. Compile RTL and Testbench files ---

vlog -reportprogress 300 ./ram.v

vlog -reportprogress 300 ./spi_slave.v

vlog -reportprogress 300 ./spi_wrapper.v

vlog -reportprogress 300 ./spi_wrapper_tb.v

# --- 3. Load the design ---

vsim -voptargs="+acc" work.spi_wrapper_tb

# --- 4. Clear old waves and add all signals clearly ---

delete wave *

add wave -noupdate -divider {SPI Master Signals}

add wave -color Cyan /spi_wrapper_tb/clk

add wave -color Yellow /spi_wrapper_tb/rst_n

add wave -color Magenta /spi_wrapper_tb/SS_n

add wave -color Green /spi_wrapper_tb/MOSI

add wave -color Green /spi_wrapper_tb/MISO

add wave -noupdate -divider {SPI Slave FSM & Registers}

add wave -radix unsigned -color Orange /spi_wrapper_tb/uut/u_spi_slave/curr_state

add wave -radix unsigned -color Khaki /spi_wrapper_tb/uut/u_spi_slave/bit_counter

add wave -radix hexadecimal -color Pink /spi_wrapper_tb/uut/u_spi_slave/rx_shift_reg

add wave -radix hexadecimal -color Pink /spi_wrapper_tb/uut/u_spi_slave/tx_shift_reg

add wave -noupdate -divider {RAM Interface (Valid & Data)}

add wave -radix hexadecimal -color DodgerBlue /spi_wrapper_tb/uut/u_spi_slave/rx_data

add wave -color Yellow /spi_wrapper_tb/uut/u_spi_slave/rx_valid

add wave -radix hexadecimal -color DodgerBlue /spi_wrapper_tb/uut/u_spi_slave/tx_data

add wave -color Yellow /spi_wrapper_tb/uut/u_spi_slave/tx_valid

# --- 5. Run simulation ---

run -all

# --- 6. Zoom to fit whole simulation wave ---

wave zoomfull

## 2. TB

`timescale 1ns / 1ps

module spi_wrapper_tb;

reg clk;

reg rst_n;

reg SS_n;

reg MOSI;

wire MISO;

integer error_count = 0;

integer test_count  = 0;

// Instantiate Top Level Module (Wrapper)

spi_wrapper uut (

.clk(clk),

.rst_n(rst_n),

.SS_n(SS_n),

.MOSI(MOSI),

.MISO(MISO)

);

// Clock Generation (100MHz -> 10ns period)

always #5 clk = ~clk;

// Task 1: Send 10 bits serially over MOSI

task send_10bit(input [9:0] data_in);

integer i;

begin

@(negedge clk);

#1;

SS_n = 1'b0;          // Activate slave

for (i = 9; i >= 0; i = i - 1) begin

MOSI = data_in[i];

@(negedge clk);

#1;

end

end

endtask

// Task 2: Read data from MISO with aligned sampling

task receive_and_check_8bit(input [7:0] expected_data);

integer i;

reg [7:0] rx_byte;

begin

rx_byte = 8'h00;

repeat(2)   @(posedge clk);

#1;

for (i = 7; i >= 0; i = i - 1) begin

rx_byte[i] = MISO;

if (i > 0) begin

@(posedge clk);

#1;

end

end

@(negedge clk);

#1;

SS_n = 1'b1;

test_count = test_count + 1;

if (rx_byte === expected_data) begin

$display("\033[1;32m [PASS] Test %0d: Expected = 0x%0h | Read = 0x%0h \033[0m", test_count, expected_data, rx_byte);

end else begin

$display(" [FAIL] Test %0d: Expected = 0x%0h | Read = 0x%0h", test_count, expected_data, rx_byte);

error_count = error_count + 1;

end

repeat (2) @(negedge clk);

#1;

end

endtask

// Task 3: Complete SPI Transaction Sequence

task run_spi_transaction(input [7:0] addr, input [7:0] write_data);

begin

$display("\033[1;36m\n---> Transaction: Addr=0x%0h Data=0x%0h\033[0m", addr, write_data);

send_10bit({2'b00, addr});

SS_n = 1'b1;

repeat (2) @(negedge clk); #1;

send_10bit({2'b01, write_data});

SS_n = 1'b1;

repeat (2) @(negedge clk); #1;

send_10bit({2'b10, addr});

SS_n = 1'b1;

repeat (2) @(negedge clk); #1;

send_10bit(10'b11_00000000);

receive_and_check_8bit(write_data);

end

endtask

// Initial Block - Explicit 25 Directed Test Cases Execution

initial begin

clk   = 0;

rst_n = 0;

SS_n  = 1;

MOSI  = 0;

#25;

rst_n = 1;

#25;

$display("=================================");

$display("   STARTING AUTOMATED SELF-CHECKING TB        ");

$display("===============================");

// LEVEL 1: BASIC SANITY TESTS

$display("\n---> [LEVEL 1] Basic Sanity Tests");

run_spi_transaction(8'h01, 8'h10); // Test 1

run_spi_transaction(8'h02, 8'h20); // Test 2

run_spi_transaction(8'h05, 8'h0F); // Test 3

run_spi_transaction(8'h0A, 8'h05); // Test 4

run_spi_transaction(8'h0F, 8'h33); // Test 5

// LEVEL 2: CORNER BOUNDARIES

$display("\n---> [LEVEL 2] Corner Boundaries");

run_spi_transaction(8'h00, 8'h00); // Test 6

run_spi_transaction(8'hFF, 8'hFF); // Test 7

run_spi_transaction(8'h00, 8'hFF); // Test 8

run_spi_transaction(8'hFF, 8'h00); // Test 9

run_spi_transaction(8'h01, 8'h80); // Test 10

run_spi_transaction(8'h80, 8'h01); // Test 11

run_spi_transaction(8'h7F, 8'h80); // Test 12

// LEVEL 3: SIGNAL TOGGLING

$display("\n---> [LEVEL 3] Signal Toggling");

run_spi_transaction(8'hAA, 8'h55); // Test 13

run_spi_transaction(8'h55, 8'hAA); // Test 14

run_spi_transaction(8'hC3, 8'h3C); // Test 15

run_spi_transaction(8'hF0, 8'h0F); // Test 16

run_spi_transaction(8'hE7, 8'h18); // Test 17

run_spi_transaction(8'h81, 8'h7E); // Test 18

run_spi_transaction(8'h69, 8'h96); // Test 19

// LEVEL 4: STRESS TESTS

$display("\n---> [LEVEL 4] Stress Tests");

run_spi_transaction(8'h13, 8'hA7); // Test 20

run_spi_transaction(8'hB4, 8'h2E); // Test 21

run_spi_transaction(8'h4D, 8'hB2); // Test 22

run_spi_transaction(8'h9A, 8'h6C); // Test 23

run_spi_transaction(8'hDE, 8'hAD); // Test 24

run_spi_transaction(8'hBE, 8'hEF); // Test 25

// FINAL SUMMARY REPORT

#100;

$display("\n================================");

$display("          TEST SUMMARY RESULT                ");

$display("===============================================");

$display(" Total Tests Executed : %0d", test_count);

if (error_count == 0) begin

$display(" STATUS           : ALL 25 TESTS PASSED! 🎉");

end else begin

$display(" STATUS  : FAILED WITH %0d ERRORS ❌",error_count);

end

$display("===========================================\n");

$finish;

end

endmodule

## 1. QuestaSim Simulation Waveforms

Paste the captured waveforms from QuestaSim showing inputs assigned (clk, rst_n, SS_n, MOSI) and outputs visible (MISO, internal signals). Ensure write and read state transitions are clear.

[ PASTE SCREENSHOT HERE: 1. QuestaSim Simulation Waveforms - Image 1 ]

[ PASTE SCREENSHOT HERE: 1. QuestaSim Simulation Waveforms - Image 2 ]

[ PASTE SCREENSHOT HERE: 1. QuestaSim Simulation Waveforms - Image 3 ]

[ PASTE SCREENSHOT HERE: 1. QuestaSim Simulation Waveforms - Image 2 ] # ================================================== # STARTING AUTOMATED SELF-CHECKING TB # ================================================== # ---> Starting Transaction for Addr: 0xa5 ¦ Data: 0x3c # [PASS] Test 1: Read Data Match! Expected = 0x3c ¦ Read = 0x3c # # ---> Starting Transaction for Addr: 0x1f ¦ Data: 0x89 # [PASS] Test 2: Read Data Match! Expected = 0x89 ¦ Read = 0x89 # # ---> Starting Transaction for Addr: 0x7e ¦ Data: 0xf0 # [PASS] Test 3: Read Data Match! Expected = 0xf0 ¦ Read = 0xf0 # ---> Starting Transaction for Addr: 0xa5 ¦ Data: 0x3c # --- [SAMPLING START] Reading MISO for Expected: 0x3c --- # -> Bit[7] at t=556000 ns ¦ MISO = 0 # -> Bit[6] at t=566000 ns ¦ MISO = 0 # -> Bit[5] at t=576000 ns ¦ MISO = 1 # -> Bit[4] at t=586000 ns ¦ MISO = 1 # -> Bit[3] at t=596000 ns ¦ MISO = 1 # -> Bit[2] at t=606000 ns ¦ MISO = 1 # -> Bit[1] at t=616000 ns ¦ MISO = 0 # -> Bit[0] at t=626000 ns ¦ MISO = 0 # --- [SAMPLING END] Assembled Byte = 0x3c --- # # [PASS] Test 1: Read Data Match! Expected = 0x3c ¦ Read = 0x3c # # ---> Starting Transaction for Addr: 0x1f ¦ Data: 0x89 # --- [SAMPLING START] Reading MISO for Expected: 0x89 --- # -> Bit[7] at t=1166000 ns ¦ MISO = 1 # -> Bit[6] at t=1176000 ns ¦ MISO = 0 # -> Bit[5] at t=1186000 ns ¦ MISO = 0 # -> Bit[4] at t=1196000 ns ¦ MISO = 0 # -> Bit[3] at t=1206000 ns ¦ MISO = 1 # -> Bit[2] at t=1216000 ns ¦ MISO = 0 # -> Bit[1] at t=1226000 ns ¦ MISO = 0 # -> Bit[0] at t=1236000 ns ¦ MISO = 1 # --- [SAMPLING END] Assembled Byte = 0x89 --- # # [PASS] Test 2: Read Data Match! Expected = 0x89 ¦ Read = 0x89 # # ---> Starting Transaction for Addr: 0x7e ¦ Data: 0xf0 # --- [SAMPLING START] Reading MISO for Expected: 0xf0 --- # -> Bit[7] at t=1776000 ns ¦ MISO = 1 # -> Bit[6] at t=1786000 ns ¦ MISO = 1 # -> Bit[5] at t=1796000 ns ¦ MISO = 1 # -> Bit[4] at t=1806000 ns ¦ MISO = 1 # -> Bit[3] at t=1816000 ns ¦ MISO = 0 # -> Bit[2] at t=1826000 ns ¦ MISO = 0 # -> Bit[1] at t=1836000 ns ¦ MISO = 0 # -> Bit[0] at t=1846000 ns ¦ MISO = 0 # --- [SAMPLING END] Assembled Byte = 0xf0 --- # # [PASS] Test 3: Read Data Match! Expected = 0xf0 ¦ Read = 0xf0 # # ================================================== # TEST SUMMARY RESULT # ================================================== # Total Tests Executed : 3 # STATUS : ALL TESTS PASSED SUCCESSFULLY! ðŸ‰ # ================================================== Using alternate file: ./wlfteiwxhn # ================================================== # STARTING AUTOMATED SELF-CHECKING TB # (25 DIRECTED & EXTENSIVE TEST CASES) # ================================================== # # ---> [LEVEL 1] Basic Sanity & Low-Value Tests # [1;36m # ---> Starting Transaction for Addr: 0x1 ¦ Data: 0x10[0m # [1;32m [PASS] Test 1: Read Data Match! Expected = 0x10 ¦ Read = 0x10 [0m # [1;36m # ---> Starting Transaction for Addr: 0x2 ¦ Data: 0x20[0m # [1;32m [PASS] Test 2: Read Data Match! Expected = 0x20 ¦ Read = 0x20 [0m # [1;36m # ---> Starting Transaction for Addr: 0x5 ¦ Data: 0xf[0m # [1;32m [PASS] Test 3: Read Data Match! Expected = 0xf ¦ Read = 0xf [0m # [1;36m # ---> Starting Transaction for Addr: 0xa ¦ Data: 0x5[0m # [1;32m [PASS] Test 4: Read Data Match! Expected = 0x5 ¦ Read = 0x5 [0m # [1;36m # ---> Starting Transaction for Addr: 0xf ¦ Data: 0x33[0m # [1;32m [PASS] Test 5: Read Data Match! Expected = 0x33 ¦ Read = 0x33 [0m # # ---> [LEVEL 2] Corner Boundaries & Walking Bit Patterns # [1;36m # ---> Starting Transaction for Addr: 0x0 ¦ Data: 0x0[0m # [1;32m [PASS] Test 6: Read Data Match! Expected = 0x0 ¦ Read = 0x0 [0m # [1;36m # ---> Starting Transaction for Addr: 0xff ¦ Data: 0xff[0m # [1;32m [PASS] Test 7: Read Data Match! Expected = 0xff ¦ Read = 0xff [0m # [1;36m # ---> Starting Transaction for Addr: 0x0 ¦ Data: 0xff[0m # [1;32m [PASS] Test 8: Read Data Match! Expected = 0xff ¦ Read = 0xff [0m # [1;36m # ---> Starting Transaction for Addr: 0xff ¦ Data: 0x0[0m # [1;32m [PASS] Test 9: Read Data Match! Expected = 0x0 ¦ Read = 0x0 [0m # [1;36m # ---> Starting Transaction for Addr: 0x1 ¦ Data: 0x80[0m # [1;32m [PASS] Test 10: Read Data Match! Expected = 0x80 ¦ Read = 0x80 [0m # [1;36m # ---> Starting Transaction for Addr: 0x80 ¦ Data: 0x1[0m # [1;32m [PASS] Test 11: Read Data Match! Expected = 0x1 ¦ Read = 0x1 [0m # [1;36m # ---> Starting Transaction for Addr: 0x7f ¦ Data: 0x80[0m # [1;32m [PASS] Test 12: Read Data Match! Expected = 0x80 ¦ Read = 0x80 [0m # # ---> [LEVEL 3] High-Frequency Signal Toggling (Noise & Crosstalk) # [1;36m # ---> Starting Transaction for Addr: 0xaa ¦ Data: 0x55[0m # [1;32m [PASS] Test 13: Read Data Match! Expected = 0x55 ¦ Read = 0x55 [0m # [1;36m # ---> Starting Transaction for Addr: 0x55 ¦ Data: 0xaa[0m # [1;32m [PASS] Test 14: Read Data Match! Expected = 0xaa ¦ Read = 0xaa [0m # [1;36m # ---> Starting Transaction for Addr: 0xc3 ¦ Data: 0x3c[0m # [1;32m [PASS] Test 15: Read Data Match! Expected = 0x3c ¦ Read = 0x3c [0m # [1;36m # ---> Starting Transaction for Addr: 0xf0 ¦ Data: 0xf[0m # [1;32m [PASS] Test 16: Read Data Match! Expected = 0xf ¦ Read = 0xf [0m # [1;36m # ---> Starting Transaction for Addr: 0xe7 ¦ Data: 0x18[0m # [1;32m [PASS] Test 17: Read Data Match! Expected = 0x18 ¦ Read = 0x18 [0m # [1;36m # ---> Starting Transaction for Addr: 0x81 ¦ Data: 0x7e[0m # [1;32m [PASS] Test 18: Read Data Match! Expected = 0x7e ¦ Read = 0x7e [0m # [1;36m # ---> Starting Transaction for Addr: 0x69 ¦ Data: 0x96[0m # [1;32m [PASS] Test 19: Read Data Match! Expected = 0x96 ¦ Read = 0x96 [0m # # ---> [LEVEL 4] Advanced Stress & Asymmetric Data Patterns # [1;36m # ---> Starting Transaction for Addr: 0x13 ¦ Data: 0xa7[0m # [1;32m [PASS] Test 20: Read Data Match! Expected = 0xa7 ¦ Read = 0xa7 [0m # [1;36m # ---> Starting Transaction for Addr: 0xb4 ¦ Data: 0x2e[0m # [1;32m [PASS] Test 21: Read Data Match! Expected = 0x2e ¦ Read = 0x2e [0m # [1;36m # ---> Starting Transaction for Addr: 0x4d ¦ Data: 0xb2[0m # [1;32m [PASS] Test 22: Read Data Match! Expected = 0xb2 ¦ Read = 0xb2 [0m # [1;36m # ---> Starting Transaction for Addr: 0x9a ¦ Data: 0x6c[0m # [1;32m [PASS] Test 23: Read Data Match! Expected = 0x6c ¦ Read = 0x6c [0m # [1;36m # ---> Starting Transaction for Addr: 0xde ¦ Data: 0xad[0m # [1;32m [PASS] Test 24: Read Data Match! Expected = 0xad ¦ Read = 0xad [0m # [1;36m # ---> Starting Transaction for Addr: 0xbe ¦ Data: 0xef[0m # [1;32m [PASS] Test 25: Read Data Match! Expected = 0xef ¦ Read = 0xef [0m # # ================================================== # [1;33m TEST SUMMARY RESULT [0m # ================================================== # Total Tests Executed : 25 # [1;32m STATUS : ALL 25 TESTS PASSED SUCCESSFULLY! ðŸ‰ [0m # ==================================================

## 2. QuestaLint Code Quality Analysis

Paste the snippet from QuestaLint proving that the design contains NO linting errors or warnings.

[ PASTE SCREENSHOT HERE: 2. QuestaLint Code Quality Analysis - Image 1 ]

## 3. Synthesis Results & FSM Encodings Comparison

Include RTL Schematic post-elaboration/synthesis, synthesis reports showing the chosen FSM encoding (Sequential, One-Hot, Gray), and highlighted Critical Path in schematic for each encoding.

[Schematic post-elaboration - Image 1 ] U_rumU_spi_slave

Image 2

## 3. Synthesis Results & FSM Encodings Comparison

Include RTL Schematic post-elaboration/synthesis, synthesis reports showing the chosen FSM encoding (Sequential, One-Hot, Gray), and highlighted Critical Path in schematic for each encoding.

[ PASTE SCREENSHOT HE 3. Synthesis Results & FSM Encodings One hot Comparison - Image 1 ] Schematic U_spi_slave ¦ U_ram

[ PASTE SCREENSHOT HERE: 3. Synthesis Results & FSM Encodings Comparison - Image 2 ] Timing Summary (Report Timing Summary)

[ PASTE SCREENSHOT HERE: 3. Synthesis Results & FSM Encodings Comparison - Image 3 ] Utilization Summary

[ PASTE SCREENSHOT HERE: 3. Synthesis Results & FSM Encodings Sequential Comparison - Image 2 ] Timing Summary (Report Timing Summary) Utilization Summary

[ PASTE SCREENSHOT HERE: 3. Synthesis Results & FSM Encodings Gray Comparison - Image 2 ] Timing Summary (Report Timing Summary) Utilization Summary

## 4. Implementation & Timing Reports

Paste the Resource Utilization Report, Timing Summary (Setup/Hold Slack), and FPGA Device View for each encoding configuration.

[ PASTE SCREENSHOT HERE: 4. Implementation One hot & Timing Reports - Image 1 ] Schematic Timing Summary (Report Timing Summary) Utilization Summary

[ PASTE SCREENSHOT HERE: 4. Implementation Sequential & Timing Reports - Image 2 ] Schematic Timing Summary (Report Timing Summary) Utilization Summary

[ PASTE SCREENSHOT HERE: 4. Implementation Gray & Timing Reports - Image 3 ] Schematic Timing Summary (Report Timing Summary) Utilization Summary

## 1. Timing Analysis Results (Post-Implementation):

## After running the complete Vivado implementation flow for each encoding method, the following Worst Negative Slack (WNS) and Worst Hold Slack (WHS) results were achieved:

## One-Hot Encoding:

## Worst Negative Slack (WNS) = 6.031 ns

## Worst Hold Slack (WHS) = 0.071 ns

## Sequential Encoding:

## Worst Negative Slack (WNS) = 6.216 ns

## Worst Hold Slack (WHS) = 0.101 ns

## Gray Encoding:

## Worst Negative Slack (WNS) = 6.306 ns

## Worst Hold Slack (WHS) = 0.131 ns

## 5. Vivado Console & Messages Tab Verification

Paste the clear screenshot of the 'Messages' tab showing SUCCESSFUL Bitstream generation with NO critical warnings or errors across Elaboration, Synthesis, and Implementation.

[ PASTE SCREENSHOT HERE: 5. Vivado Console & Messages Tab Verification - Image 1 ]

Post-Debug & Bitstream Generation:

- Shows the integrated ILA core and probe nets successfully added to the design, with the Messages tab confirming a clean run with zero critical warnings and zero errors leading to a successful bitstream generation.